# Near Surface Geophysics Glossary

## Environmental and Engineering Geophysical Society

### November 2006 Version

The following are data for a glossary on near surface geophysics with the intent of

- improving communication between geophysicists and engineers,

- providing a reference for people new to the field, and

- a quick resource for geophysicists outside their specialty.

It is planned to be a dynamic document which can be frequently upgraded.

The descriptions are intended to give the reader a short, practical concept about many of the terms commonly used in geophysics, not to give rigorous definitions.  Many terms that are common knowledge to a technically trained person have not been included.

If you have corrections, additions, or suggestions, please email them to Paul Wolfe at Wright State University.          paul.wolfe@wright.edu

The addition of geotechnical engineering terms with descriptions would be particularly appreciated.

Several people have helped in developing this Near Surface Geophysics Glossary, particularly Chuck Young of Michigan Tech.  Thanks to all who have helped and to those who submit improvements.

Paul Wolfe, Emeritus Professor, Wright State University, Dayton, Ohio

This glossary is written in Microsoft Word.  You are encouraged to download it to you computer for quick access.  Check back to the EEGS web site occasionally for an updated version.

You can go directly to any letter in the glossary by:

1.  on a PC - holding down the control key and clicking on a letter in the list below.

2.  on a Mac with Word - clicking on a letter in the list below.

3.  on UNIX with Word - clicking on a letter in the list below.

You can use the Find tool in Word’s Edit menu to lock for specific terms or words within the descriptions.

A B C D E F G H I J K L M N O P Q R S T U V W X Y Z

| # A ABEM | Geophysical equipment company based in Sundbyberg, Sweden |
| --- | --- |
| absorption | The process by which energy, signal, a chemical, etc. Is removed from a wave, flow, solution, etc. |
| absorption coefficient | A numerical measure of the rate at which absorption occurs |
| AC | Alternating current |
| accelerated weight drop | A seismic energy source that produces waves by the impact of a falling object with the ground, where the rate of fall is enhanced by a mechanical or pneumatic device |
| accelerometer | A transducer that produces an electrical signal that is proportional to the acceleration |
| acidic rock | A rock containing more than 66% SiO2 |
| acoustic | An adjective referring to properties or devices that depend on pressure waves in or near the audible range of about 20 to 20,000 Hz |
| acoustic impedance | A seismic wave parameter that is the product of density and seismic wave speed (Z=density * V) |
| acoustic log | A well log that displays a continuous record of the seismic velocity of the rocks in the borehole wall as a function of depth; also called sonic log or continuous velocity log |
| acoustic wave | A propagating pressure wave, frequently used as an approximation to a full seismic wave |
| adit | A horizontal entry into an underground mine |
| adsorption | The process of removal of a chemical or other substance from a fluid by attachment of the species to solid surfaces in the transmitting material |
| AEM | Airborne electromagnetics |
| aeromagnetic | A magnetic survey conducted from an aircraft |
| AFMAG | Audio-frequency magnetic method |
| AGC | Automatic gain control, a method for reducing the range of amplitude of a signal by scaling in a sliding data window |
| air gun | A source that produces seismic waves by rapidly releasing high pressure from an underwater chamber |
| air wave | An acoustic wave that propagates through the air from a seismic source |
| aliasing | An effect of producing false frequency components in digitally sampled data when the sampling rate is lower than the Nyquist frequency |
| alkali-vapor magnetometer | A magnetometer that relies on the dependence of energy levels in alkali atoms on the magnetic field |
| alluvium | Deposits made by streams in its channel, in its delta and on its floodplain, usually unconsolidated |
| altitude | The vertical height above a reference level, usually mean sea level |
| AM | Amplitude modulated |
| ambiguity | The fact that a number of different, sometimes drastically different, geological material distributions or models can produce identical or very similar geophysical responses |
| ampere | The basic unit of electrical current flow; 1 ampere = 1 coulomb per second |
| amplitude | The maximum disturbance from equilibrium of a physical variable representing a wave; e.g., for an electromagnetic wave with amplitude Emax, the electric field oscillates between +Emax and –Emax. |
| amplitude spectrum | The distribution of the amplitudes of the Fourier components of a signal as a function of frequency |
| angle of incidence | The angle between the direction of a ray incident on a surface and the normal (perpendicular) to the surface |
| angular frequency | Frequency expressed in radians per second; angular frequency = 2π*frequency in hertz |
| anion | Negatively charged ion |
| anomalous | Adjective denoting an anomaly |
| anomaly | An object, zone, or data that deviates from the typical surroundings |
| antenna | A device for radiating and/or receiving electromagnetic waves |
| antialias filter | An electrical filter to remove high frequency components of a signal that would otherwise lead to aliasing |
| anticline | A geological structure where the rock layers are highest along the axis and descend on either side |
| antiferromagnetic | A material which has sublattices of atomic spins that are perfectly aligned within each sublattice but the sublattices have antiparallel alignment resulting in zero net magnetization |
| aperture | The opening area, real or conceptual, through which signal is admitted to a measuring system |
| APEX | Manufacturer of electromagnetic instruments |
| apparent conductivity | The conductivity calculated from electrical or electromagnetic survey results on the assumption that the earth is a homogeneous, isotropic half space |
| apparent resistivity | The resistivity calculated from electrical survey results on the assumption that the earth is a homogeneous, isotropic half space |
| apparent velocity | The velocity of wave propagation that is calculated from the rate at which a disturbance is observed to move along the surface of the ground, it is greater than or equal to the true velocity of the waves in the earth |
| aquifer | A rock formation or soil zone that is saturated with ground water and has adequate permeability to produce water from a well |
| Archie's law | The formula relating electrical resistivity of a bulk material to the fluid resistivity, saturation and porosity of the material |
| array | A physical arrangement of sensor for geophysical surveying, such as electrodes or geophones |
| arrival time | The time at which a signal reaches a sensor from the time of energy input at the source |
| ART | Algebraic reconstruction technique |
| a-spacing | The distance between electrodes in a Wenner resistivity array |
| attenuation | The decrease in signal strength with travel distance through a material |
| attenuation coefficient | A numerical measure of the rate of attenuation with distance |
| Atterburg limit tests | Tests to characterize the plasticity of soil for specified water content |
| attitude | Spatial orientation |
| autocorrelation | The statistical result of correlating a signal with a replica of itself |
| automatic gain control | A method for reducing the range of amplitude of a signal, frequently referred to as AGC |
| AVA | Amplitude variation with angle, the variation in seismic reflection amplitude with the angle of incidence of the wave, commonly used to extract Poisson's ratio from seismic reflection data |
| AVO | Amplitude variation with offset, the variation in seismic reflection amplitude with the distance between source and sensor, commonly used to extract Poisson's ratio from seismic reflection data |
| azimuth | The horizontal orientation relative to north of an object or feature |
| azimuthal anisotropy | The variation of a physical property with azimuth, commonly electrical resistivity or seismic velocity |
| azimuthal resistivity | The variation of electrical resistivity with azimuth, commonly used to determine orientation of bedrock fractures |

# B

| backscatter | The effect where waves (or particles) are scattered back into the region from which the incident wave (or particles) came |
| --- | --- |
| bandpass | The range of frequencies passed by a filter or system which cuts out high and low frequencies |
| bandwidth | The range of frequency content of a signal or the range of frequencies passed by a system |
| bar | A unit of pressure equal to 105 N/m2, it is approximately one atmosphere |
| base station | A reference point that is used in a gravity or magnetic survey to correct for temporal variations in the field; a reference point which is established to allow tying to other surveys |
| baseline | A reference line which may be a line on the earth or a map for surveying for data reference or a conceptual line of reference on a well log, graph, etc. |
| basic rocks | Rocks with low silica content, defined by less than 52% SiO2 |
| bearing | A compass direction |
| bedding plane | A surface following the layering or beds in a sedimentary rock |
| bedrock | Consolidated, mechanically competent rock in place within the earth, it may be covered with soil or regolith or it may be exposed at the surface |
| benchmark | In surveying: a permanently set reference point with accurately known coordinate position. Alternate meaning: criteria against which performance is measured, such as the speed of a computer program for a specified calculation |
| Betsy Seisgun | A trade name for a seismic source that fires shotgun shells |
| BHTV | Borehole televiewer |
| bin size | The dimensions of the area used to determine which seismic traces to stack in the CMP method |
| binning | The process in seismic reflection surveying where the surface area is divided into small regions (called bins) and all reflections which have a midpoint in a bin are summed to improve the signal to noise ratio |
| bipole | A pair of electrodes with opposite polarity that are treated as an entity, if the spacing is small for the scale of the application it is called a dipole |
| bird | A device towed behind an aircraft which contains instruments for an airborne geophysical survey |
| bistatic mode | In ground penetrating radar: a technique which uses separate transmitting and receiving antennae |
| bit | In digital electronics: the smallest piece of information, typically designated as 0 or 1; in drilling: the end of the drill assembly that cuts the rock |
| blind zone | In seismic refraction survey: an interface which does not show up as a first arrival at any source-receiver offset due to its small thickness and a seismic wave velocity which is insufficiently greater than the underlying layer. |
| blowout | An event when fluids and other materials are expelled from a borehole by the release of downhole pressure which exceeds the hydrostatic pressure of the fluid column in the hole |
| body wave | A seismic (or possibly electromagnetic) wave which propagates through the interior of a transmitting object |
| borehole gravimeter | A gravity measuring devise that can measure the relative gravitational field in a borehole |
| borehole log | A graphical record of the materials and/or their properties as a function of depth in a borehole |
| Bouguer anomaly | A result of a gravity survey after tidal, latitude, free air, and Bouguer corrections have been performed. It is commonly the final result that is interpreted for a gravity survey. |
| Bouguer correction | A correction to gravity data that account for the gravitational attraction of a horizontal slab of rock between the station elevation and the datum elevation |
| boulder | A somewhat smoothed piece of rock which is greater than 256 mm in diameter |
| bound water | Water in a formation which is chemically bonded to constituents of the rock |
| breccia | A clastic rock consisting largely of angular or subangular fragments of pre-existing rock |
| bright spot | On seismic reflection data: a zone of abnormally high reflection amplitude |
| broadside orientation | A survey where the energy source is significantly offset perpendicular to the line of sensors |
| brownfields | Real property whose redevelopment may be complicated by the presence or potential presence of pollution. |
| Brunton compass | A precision compass used for geological field that is manufactured by the Brunton Company |
| buffalo gun | A seismic energy source used in shallow surveys; it consists of a very short barrel in which a shotgun shell is fired within a shallow borehole |
| bulk modulus | An elastic modulus that gives the negative of the ratio of the hydrostatic stress to the volume strain |
| buried valley | A bedrock valley which has been largely filled with unconsolidated material, commonly a preglacial valley filled with glacial deposits. |
| byte | A small group of digital bits that are treated as an entity, usually 8 bits that represents one character |

# C

| calcareous | Adjective for a rock containing calcium carbonate |
| --- | --- |
| calibrate | Adjust an instrument to established standards for accurate, reproducible readings |
| caliche | A hard layer that forms in the soil in arid regions due to deposition of calcium carbonate cement between soil grains |
| caliper log | A devise for logging a bore hole which measures the diameter of the hole continuously along its length |
| cap | See detonator cap |
| capillary zone | The zone above the water table in soil which has fractional water saturation do to the capillary effect |
| casing | The pipe which is slid down a bore hole to keep the hole from collapsing, usually made of steel or PVC |
| cathodic currents | Electrical currents used to protect pipelines from corrosion, these can interfere with geophysical surveys |
| cavitation | Bubble effect cause by rapid reduction of pressure due to intense waves |
| CDP | Common depth point, commonly synonymous with CMP |
| cesium magnetometer | A devise for measuring the Earth's magnetic field which utilizes the magnetic field dependence of the atomic energy levels in cesium |
| chain | A unit of length sometimes used in land surveying, 1 chain = 66 feet = 20.12 meters |
| channel wave | A wave which propagates in a low velocity layer (channel) between two higher velocity materials, the energy is significantly contained in the layer and spreads in two dimensions rather than three; the waves are dispersive |
| chargeability | A measure of the time-domain induced polarization related to secondary to primary voltage ratios. |
| chirp | A swept frequency signal |
| chlorinated organic solvents | A group of chemical that were commonly used as industrial solvents and degreasers that are a serious water pollution problem |
| clipped | A signal with amplitude larger than can be handled by the recording device and consequently has a maximum which corresponds to the device’s saturation level |
| CMP | Common mid point |
| cobble | A somewhat smoothed piece of rock with a diameter ranging from 64 mm to 256 mm |
| coherence | A measure of the similarity of 2 or more wave forms |
| colatitude | The complement of latitude, i.e., the degrees of latitude measured from the pole rather than the equator |
| Cole-Cole | A method of characterizing the time dependence of induced polarization effects using an equivalent circuit |
| common depth point | A technique of seismic reflection data acquisition, more appropriately called common mid point |
| common mid point | A technique of seismic reflection data acquisition where traces having different ray paths but the same mid point between source and receiver are summed after adjustments for geometry |
| common offset | A group of seismic traces which have the same source to receiver distance |
| competent | An adjective describing an earth material which has adequate strength to hold together under moderate mechanical disturbance |
| complex conductivity | The ratio of the current to the voltage when they are not in phase, used in induced polarization methods |
| complex resistivity | The ratio of the voltage to the current when they are not in phase, used in induced polarization methods |
| compressional waves | A longitudinal mechanical wave that propagates compressions and dilations of the material |
| cone penetrometer | An instrument for studying soil by pushing an instrumented probe with a conical tip deep into the ground |
| conglomerate | A clastic rock consisting largely of well-rounded fragments |
| constant-spacing traverse | A resistivity technique where the spacing between electrodes is held constant and the electrode set is moved at fixed steps along a line on the ground surface |
| contamination | Any physical, chemical, biological, or radiological substance or matter that has an adverse effect on air, water, or soil. |
| contrast | The difference in material properties, commonly for a body of interest to the background geology |
| converted wave | A seismic wave produced by reflection (or refraction) which is of a different mode than the incident wave, commonly it is a vertically polarized shear wave (SV) produced by an incident compressional (P) wave. |
| convolution | A mathematical procedure for determining the output of a linear system for a specified input signal |
| core | A long tube of earth material produced by drilling with a hollow drill bit |
| corner reflector | An optical devise which reflects a light beam directly back in the direction from which it came |
| correlation | Determination of the relationship of two features such as rock units or geophysical signal responses; a mathematical measure of the similarity of two signals |
| Coulomb's law | The basic physical law of the force between 2 electrical charges |
| CPT | Cone electrometer - a devise which makes subsurface measurements by pushing a calibrated probe into the earth |
| critical angle | When a wave strikes an interface where the wave propagation velocity increase |
| critical distance | In seismic refraction, the distance from the source at which reflection and refraction times are the same; this corresponds to the onset of the head wave arrival |
| critical refraction | Waves that refract at the critical angle |
| cross correlation | A measure of the degree of linear relationship between 2 signals |
| cross-hole tomography | A technique for determining the distribution of physical properties of the zone between two bore holes by making an extensive series of measurement with the source(s) in one hole and the receiver(s) in the other; this applies to many techniques such as resistivity, seismic, radar, etc. |
| cross-line | An adjective describing something that is perpendicular to the direction of a survey line |
| crossover distance | In seismic refraction it is the distance at which the head waves from the first refracting layer arrive at the same time as the direct waves |
| CSAMT | Controlled source audio magnetotelluric method |
| cultural noise | The response of a geophysical instrument to man-made objects, devices, or activities which interfere with measuring the desired signal |
| Curie temperature | The maximum temperature at which a ferromagnetic material can remain ferromagnetic; variable, but in the neighborhood of 550 degree Celsius for a number of typical materials |
| current | The flow of electric charges, measured in amperes |
| current density | The current flow per unit area through a small surface area, measured in amperes/square meter |
| current distribution | The distribution of current density through the earth as it flows between positive and negative electrodes |
| current electrode | A devise, such as a metal stake, to act as an electrical current source or sink in the earth |
| curve-matching techniques | A resistivity interpretation technique which uses log-log graphs of apparent resistivity versus electrode spacing to com pare with theoretical curves; similar techniques can be used with some other electrical and electromagnetic methods |

# D

| damping ratio | A measure of vibrational energy dissipation commonly used by engineers, equal to 1/(2Q) |
| --- | --- |
| darcy | A unit of permeability; 1 darcy = 1 centipoise cm2/(atmosphere sec) = 9.87x10-9 cm2 |
| Darcy's law | The law for fluid flow through a permeable material with flow rate proportional to pressure gradient |
| datum | A reference level, usually an elevation, to which measurements are referenced, a common example is the sea-level datum |
| dB | Abbreviation for decibel |
| DC | Direct current; a constant voltage component of a signal |
| decade | A factor of 10 |
| decay time | The characteristic time for a decaying signal or radioactive material to decrease to 1/e of its earlier value |
| decibel | A logarithmic unit for comparison of signal strength or power, given by 10 log (P/P0) for energy |
| decimation | Reducing the sampling rate of a digitized signal by keeping only every tenth, fourth, etc. Sample |
| declination | The angle between true north and magnetic north at a specified location, positive if a compass point east of true north |
| deconvolution | The mathematical procedure for removing the response of a linear system from the output signal to find the input signal |
| degauss | Process of removing the permanent magnetization of a sample |
| delay-time method | A seismic refraction interpretation method for irregular refracting surfaces that estimates the depth from time difference between waves arriving at the surface and the corresponding travel time along the refractor |
| delta function | An idealized mathematical function that is zero everywhere except where the argument equals 0; for a continuous function it is sharply peaked with an area of unity(also called Dirac delta function), for a digital function it has a value of one where the argument equals 0 |
| demagnetization effect | The modification of the magnetic field in a body due to the shape of the body |
| demultiplex | The process of sorting out different signals that were mixed into a single data stream |
| density contrast | The difference in density between the body of interest and the surrounding material |
| depth map | A map of the topography expressed in depth units of a surface defined by a physical property of the material |
| depth of investigation | The maximum depth at which a geophysical technique provides useful information about the subsurface; it depends on the technique, the local conditions, and the type of target |
| depth of penetration | The depth to which currents or fields used for exploration penetrate the subsurface, closely related to depth of investigation |
| depth section | A seismic section where the vertical scale has been converted to depth using a velocity function; Similar sections for other geophysics survey techniques |
| depth section | A cross section expressed in depth units to a surface defined by a physical property of the material |
| detonating cord | Explosive cord that is used as a seismic energy source or for interconnecting other explosive charges |
| detonator caps | In explosives - a small explosive devise which is detonated by a pulse of electrical current and is used to detonate a larger explosive |
| DGPS | Differential global positioning system |
| diagenesis | The physical and chemical processes of altering sedimentary materials after they have been deposited |
| diamagnetic | A type of magnetic material which acquires a very weak induced magnetization opposite to the applied magnetic field |
| dielectric constant | The electrical property of a material that give a measure of its ability to store charge |
| differential compaction | Compaction of a group of sedimentary materials that varies in space due to differences in properties or external forces |
| diffraction | The spreading of a wave front when it strikes a discontinuity in the propagating medium |
| diffusion | The mixing of one material (usually fluid) into another by random molecular transport; the spreading of a signal through a volume of space |
| dike | Geology: a igneous, intruded tabular body that cuts across bedding; Engineering: a surface water control structure which is controls the lateral extent of water flow, usually long and relatively narrow |
| dilatational wave | A seismic wave or other mechanical wave where the particle motion is along the line of wave propagation; also called P waves and compressional waves |
| dip | The angle that a surface or linear feature makes with a horizontal plane |
| dipole | A pair of electrodes, one positive and one negative, that are close together compared to the scale of the application in which they are used |
| dipole-dipole array | An arrangement of an electrical or electromagnetic survey where the source and receiver are both characterized by as dipoles and are separated by a distance significantly longer than the size of the dipoles |
| dipole field | The characteristic electric or magnetic field produced by an electric or magnetic dipole; it possesses rotational symmetry around the dipole axis |
| dipping interface | A surface separating to earth materials when that surface is not horizontal |
| Dirac delta function | See delta function |
| direct shear test | Common laboratory test to measure shear strength of soil as a function of normal stress |
| direct wave | A seismic or electromagnetic wave that propagate direct from the source to the receiver in a approximately a straight line |
| dispersion | Spreading of a material or field in space; this term has many uses including molecular dispersion, spreading of a wave form due to velocity dependence on wavelength, transient electric field spreading in the subsurface, etc. |
| displacement | The linear distance of the physical movement of a body from one place to another |
| distal | The part of a sedimentary deposit that is farther from the source of the sediments |
| diurnal | Adjective describing repetitive daily cycle |
| diving wave | A seismic wave that is continually refracted as the ray propagates through a strong velocity-gradient medium so the wave bends and eventually returns to the surface |
| divining | Purporting to locate water or oil deposits by magical methods, typically forked twigs |
| Dix equation | An equation relating the interval seismic velocity of a layer to the RMS velocity of waves reflected from the top and the bottom interfaces of the layer |
| DNAPL | Dense non-aqueous phase liquids, contaminants which are denser than water |
| doline | Sinkhole |
| domain | A zone in a magnetic mineral grain where the magnetic dipole moments of the atoms are aligned |
| Doppler effect | The shift in frequency of a wave when there is motion of the source and observer |
| downward continuation | A mathematical technique for using the field equations to calculate the field at a deeper level from know values at a higher level, becomes unstable near anomalous bodies and for noisy data |
| dowser | A person who practices divining for water |
| drift | A gradual variation in field or signal that is not related to the physical property being measured |
| drilling mud | A suspension of fine particles in water that is used in rotary drilling to cool the drill bit, remove cuttings from the borehole and to keep fluids in the penetrated rocks from entering the hole |
| dynamic range | The ratio of the maximum to the minimum (non-zero, frequently noise level) signal that a measuring system can record; for digital systems it equals 2n-1 where n is the number of bits of the digitized signal |

# E

| EEGS | Environmental and Engineering Geophysical Society |
| --- | --- |
| effective stress | Soil mechanics: total stress minus porewater pressure |
| Eh | Oxidation potential |
| elastic | Implies zero net displacement upon release of an imposed force |
| elastic constants | Properties of a material that describe its mechanical properties such as the proportionality between stress and strain |
| elastic moduli | Elastic constants |
| elastic wave | A wave that propagates by elastic deformation of the transmitting material |
| electric profiling | A resistivity technique with a fixed configuration of electrodes that is advanced stepwise along a traverse |
| electric sounding | A resistivity technique with the center of the configuration held at a fixed location and the spacing of electrodes increased stepwise (usually in a geometric progression) to increase the depth of investigation |
| electrical conductivity | The proportionality constant of a material that relates the current density to the applied electric field, commonly the property determined in an electromagnetic survey |
| electrical resistivity | The inverse of electrical conductivity, the physical property determined in a resistivity survey |
| electrochemical effect | An electrical response to chemical activity |
| electrokinetic | The electrical response produced when a fluid is forced to flow through a permeable material |
| electromagnetic sounding | An electromagnetic survey technique where progressively deeper layers of the earth are investigated beneath a fixed surface location |
| electromagnetic survey | A geophysical survey method that normally determines the distribution of electrical conductivity on the basis of currents induced by time varying magnetic fields |
| electromotive force | The electric potential difference produced by a source such as a battery |
| EM | Electromagnetic |
| emf | Electromotive force |
| energy partition | The relative energy in the various outgoing seismic waves when an incoming wave strikes an interface |
| energy source | Seismic: the mechanism for generating seismic waves such as explosive, vibrator, or earthquake; a source of electricity to power equipment, such as a battery or generator |
| entry | A mine passage used for transport or ventilation |
| Eötvös effect | The change in the gravity effect due to the motion of the gravimeter along the surface of the earth |
| Eötvös unit | A unit of the gravity gradient equal to 1 nanogal/cm |
| epoch | A geologic time subdivision; the time at which a geophysical data calculation is accurate |
| equipotential method | A geophysical survey method that maps the lines of constant electric potential on the surface of the earth, commonly due to fluid flow through a permeable material or electro chemical action of an ore body |
| equipotential surface | A theoretical surface which is the locus of all points having the same value of potential, most commonly used for electric potential |
| equivalence | In geophysical interpretation: this the range of values for a combination of parameters such that the resulting response difference is too small to confidently detect in a practical situation |
| ERT | Electrical resistivity tomography |
| evanescent wave | A wave that diminish within a short distance of a boundary, commonly occurs when incident waves exceed the critical angle producing total reflection |
| event | Seismic: a feature, such as a peak, on a seismic section that can be recognized on several traces; an occurrence at a specific time such as an earthquake |
| expanding-spacing technique | A resistivity (or electromagnetic) procedure where the center of the electrodes (coils) is kept fixed and the spacing between them is expanded stepwise (usually in a geometric progression) to produce increasing greater depth of penetration, the purpose is to produce a depth profile of resistivity |

# F

| fabric | (a) The spatial arrangement and orientation of the components (crystals, particles, cement) of a sedimentary rock. (b) The complete spatial and geometrical configuration of all those components that make up a deformed rock. It covers such terms as texture, structure, and preferred orientation. |
| --- | --- |
| facies | A rock unit that is distinguishable in the field by its appearance, structure, and composition |
| fan shooting | A seismic refraction technique where the source is offset approximately perpendicular to the geophone spread so that the pattern of ray paths is fan shaped |
| farad (F) | The SI unit of electric capacitance. One farad is defined as the ability to store one coulomb of charge per volt of potential difference between the two conductors |
| faraday (Fd) | A unit of electric charge. One faraday is equal to the product of Avogadro’s number (see mole) and the charge (1 e) on a single electron. |
| Faraday's Law | The law of electromagnet induction which states that the voltage induced around a circuit is proportional to the time rate of change of the magnetic flux through the circuit |
| fat clay | A cohesive and compressible clay of high plasticity, containing a high proportion of minerals that make it greasy to the feel. It is difficult to work when damp, but strong when dry. |
| fault | A fracture or fracture zone where there has been relative movement between the rocks on opposite sides |
| FDEM | Frequency-domain electromagnetic |
| femto- (f-) | A metric prefix standing for 10-15 (one quadrillionth). |
| ferric | Of, pertaining to, or containing iron in the trivalent state; e.g., ferric chloride, FeCl |
| filter | A device or operation that selectively removes components of the input stream; a common type is a frequency filter which removes selected frequency components from a signal |
| first arrival | The first signal to arrive at a detector from a source input; the time of first arrival is the primary information for a seismic refraction survey |
| flops | A unit of computing power equal to one floating point operation per second. |
| flux | The rate of flow of a physical quantity through a reference surface |
| fluxgate magnetometer | An instrument for measuring the magnetic field component along the axis of a magnetically saturable core assembly |
| fold of coverage | The number of traces of seismic reflection data that are composited to form a single trace for the purpose if improving signal-to-noise ratio |
| footwall | The zone of rock below a fault |
| formation | A rock unit that is mappable |
| Formation Evaluation | The analysis of subsurface formation characteristics, such as lithology, porosity, permeability and saturation, by indirect methods such as wireline well logging or by direct methods such as mud logging and core analysis. |
| forward modeling | The process of determining the geophysical response that would result from a particular model, used in interpreting results or planning surveys |
| Fourier analysis | The mathematically procedure to separate a signal into its various frequency components |
| frame | A single image or picture. A single complete vertical scan of the cathode ray tube (CRT). |
| free-air anomaly | The result of a gravity survey after the free-air correction but before the Bouguer correction |
| free-air correction | A correction applied to gravity data that compensates for the location of the observation station at an elevation different from the reference elevation (e.g., sea level), it does not account for any rock that might be between the two elevations |
| frequency-domain electromagnetic | An electromagnetic survey technique where discrete frequencies are used, as opposed to time-domain EM |
| Fresnel zone | The zone on a reflecting surface that contributes wave components that are within one-half wavelength of the reflection from the center when they arrive at the detector |
| fundamental mode | The lowest frequency of a resonant system |

# G

| G | Giga, prefix indicating 109 in SI units |
| --- | --- |
| g | A unit of acceleration equal to the typical gravitational attraction at the surface of Earth (9.81 m/s/s) |
| g.u. | Gravity units, 0.1 milligal or 10-6 Newton/kg (or m/s2) |
| gain | The multiplicative factor by which a signal is increased by a system or systems |
| Gal | A unit of measurement of the gravitation field (10-2 newton/kg) or the acceleration of gravity (1 cm/s2) |
| gamma | A unit of magnetic field equal to the official SI unit of 1 nanotesla, also equal to 10-5 gauss |
| gamma ray | A photon of electromagnetic energy emitted by a decaying radioactive nucleus |
| gamma-ray log | A well log that reads gamma ray intensity naturally emitted from formations. Shales generally produce higher levels of gamma radiation and can be detected and studied with the gamma ray tool. |
| gamma-ray spectroscopy log | A well log that records the energy distribution of gamma ray emission as a function of depth in the well; this provides information on the radioactive species distribution in the well |
| gauss | A cgs-emu unit of magnetic field, equal to 10-4 tesla; the geomagnetic field is on the order of a gauss |
| generalized reciprocal method | A seismic refraction interpretation method that uses the "optimum XY distance" to determine the point at which upward traveling waves leave the refractor to improve the interpretation. It avoids some of the pitfalls of simpler interpretation methods but requires a more complete data set. |
| geoelectric | Adjective referring to the electrical properties of earth materials |
| geoid | A theoretical sea level surface that is extended through the continents, it is a surface of constant gravitational potential |
| geomagnetic | Adjective referring to the Earth's magnetic field |
| geophone | A device used in seismic surveys that senses ground vibrations and outputs an electrical signal that is proportional to the velocity of vibration |
| geophysics | The study of the Earth that utilizes the phenomena of physics to determine Earth properties |
| giga | G, prefix indicating 109 in SI units |
| gigahertz (GHz) | A unit of frequency equal to 109 per second, or 1 per nanosecond. Cellular phones and microwave ovens operate with radio waves having frequencies in the gigahertz range. |
| global positioning system | GPS, a system for locating positions anywhere on the surface of the earth using signal from the U.S. government satellite system |
| googol | A unit of quantity equal to 10100 |
| GPR | Ground penetrating radar, an electromagnetic method for imaging the subsurface |
| GPS | Global positioning system |
| grad or grade or gon (g or grd) | A unit of angle measurement equal to 1/400 circle, 0.01 right angle, 0.9°, or 54'. |
| graded bedding | A measure of the steepness of a slope, such as the slope of a road or a ramp. Usually stated as a percentage, the grade is the same quantity known as the slope in mathematics: the amount of (vertical) change in elevation per unit distance horizontally ("rise over run"). |
| graded bedding | A sedimentary deposit with a gradational change in grain size from course to fine [ see also well-graded] |
| gradient | The rate of change of the value of a field or property with distance; the first derivative with distance; |
| gradient array | An electrical survey method where the two potential electrodes are closely spaced to provide a measurement of the gradient of the electric field |
| gradiometer | A survey device which measures the gradient of a field; a common example is two magnetometers that are separated by a short fixed distance and that measure simultaneously to allow the determination of the magnetic field gradient in the direction defined by the line of the detectors |
| graticule | A sheet (usually transparent) with zones indicated by lines for use in integrating the gravity effect of a density model |
| gravimeter | An instrument for measuring relative gravity field values; also, gravity meter |
| gravitational acceleration | The acceleration of a freely falling body under only the force of gravity; typical value is around 9.8 m/s2; it is numerically equivalent to the value of the gravitational field in newton/meter |
| gravitational constant | The experimentally determined proportionality constant in Newton's Law of Universal gravitation; value is approximately 6.67 x 10-11 newton - meters2/kg2 |
| gravity anomaly | A localized zone of departure of the gravitational field from the average field surrounding that is interpreted as a density contrast |
| gravity effect | The vertical component of the gravity field due to a body, which is the effect observed in a gravity survey |
| gravity meter | An instrument for measuring relative gravity field values; also, gravimeter |
| gravity unit | G.u., a unit of measurement of the gravitational field, 0.1 milligal or 10-6 newton/kg (or m/s2) |
| ground penetrating radar | An electromagnetic method that emits waves in the tens of MHz to GHz range and records the waves reflected back from subsurface dielectric constant contrasts |
| ground roll | Surface waves that are generated by a seismic source; Rayleigh waves are a major component; ground roll is a serious noise component that can obscure portions of seismic reflection data |
| ground truth | Direct physical observation that is used to test indirect interpretations such as geophysical results |
| group interval | The distance between the centers of geophone groups |
| GRS67 | Geodetic Reference System (1967); a formula for the smoothed gravity value as a function of latitude |
| guided waves | Waves that are wholly or partially constrained to travel in a 2 dimensional layer |

# H

| half life | The time for one half of the nuclei of a radioactive species to decay |
| --- | --- |
| Hammer chart | A zone chart that overlays a topographic map to make terrain corrections to gravity data (developed by Sigmund Hammer) |
| hanging wall | The zone of rock above a fault |
| hardness | A measure of the hardness of a metal or mineral. The mohs hardness scale is used in geology to give a rough estimate of hardness by testing which minerals are able to scratch the sample |
| head | Hydrology: water pressure express as the height of a water column that would produce it |
| head wave | Seismic: the upward traveling wave that is generated by an interface when incident waves strike it at the critical angle; these are the basis for refraction surveys |
| henry (H) | The SI unit of electric inductance. An ideal coil with an inductance of one henry will produce a potential difference of one volt if the current through it is changing at the rate of one ampere per second. |
| hertz | Hz, the SI unit of frequency corresponding to cycles per second |
| hidden layer | A layer that cannot be observed as a first arrival in a seismic refraction survey because it has a lower velocity than the layer above or it is too thin in relation to the higher velocity layer below |
| high | Electronics: A digital logic state corresponding to a binary "1"; Geophysics: a zone where the parameter measured is distinctly higher than in surrounding zones |
| homogeneous | The condition of having the same properties at every point |
| horizon | Geology: an interface between two beds |
| Horizontal gradient of gravity | Or pseudo-gravity peaks over contacts between rocks of different density or magnetization. |
| Huygens' principle | The principle in wave theory that each point on a wavefront acts as a generator of the wave |
| hydraulic conductivity | The ratio of the fluid flow velocity to the pressure difference in a permeable material |
| hydrosphere | The totality of water encompassing the Earth, comprising all the bodies of water, ice, and water vapor in the atmosphere. |
| hydrophone | A sensor for seismic waves in water that produces an electrical response proportional to changes in pressure |
| hysteresis | The response of a material to a forcing function that depends on the previous history of the interaction, for a forcing function it relates to energy transformed into thermal energy |

# I

| IAGC | International Association of Geophysical Contractors |
| --- | --- |
| IGRF | International Geomagnetic Reference Field, a formula for calculating the large-scale average geomagnetic field any place on the earth, it is subtracted from data to enhance local anomalies |
| IGSN71 | International Gravity Standardization Net, a worldwide reference system for linking local gravity surveys |
| IGY | International Geophysical Year, a large international program to collect geophysical data in 1957-8 |
| image | A graphic presentation of a physical reality, for example, a photograph or a seismic section |
| image resolution | The area represented by each pixel of a satellite image. The smaller the area represented by a pixel, the more accurate and detailed the image. |
| imaginary part | The part of a complex number that is associated with i [=√(-1)] |
| impact device | A seismic source that relies on a sudden collision between a mass and the ground |
| impact-echo testing | Seismic reflection test in a drilled shaft foundation element to test for depth and continuity |
| impedance | Electrical: the ratio of alternating voltage to current which may be complex with the real part being the resistance and the imaginary part being the reactance; Seismic: acoustic impedance = density times wave propagation velocity |
| impermeable | Hydrology: not permitting water to flow through at a perceptible rate |
| impulse | A sharply peaked signal such as might result from a mechanical impact or an electrical spark |
| impulse response | The output of a system when an impulse is applied to the input |
| in situ | Latin for "in original place." Refers to measurements made at the actual location of the object or material measured. Compare remote sensing. |
| incidence, angle of | The angle between the normal and the direction of propagation for a wave striking an interface |
| inclination | The angle between a surface, line, or vector and the horizontal plane |
| induced current | An electrical current that flows in a circuit or a material as a response to a changing magnetic field |
| induced magnetization | Magnetization in a material that is produced in response to an applied magnetic field |
| induced polarization | IP, an electrical survey method that relies on the delayed electrical response of earth material after a current flowing through them is quickly terminated |
| induced voltage | A voltage in a circuit or material that is generated by a changing magnetic field, it produces an induced current in conducting material |
| in-line array | An arrangement where sources and detectors lie along a common line |
| in-phase component | The part of an observed signal that has the same phase as the input signal |
| in-seam | A phenomenon or technique that is used within a coal seam |
| IRIS | Incorporated Research Institutions for Seismology is a university research consortium dedicated to exploring the Earth's interior through the collection and distribution of seismographic data |
| Intensity of magnetization | Magnetization, M; the net magnetic dipole moment per unit volume of a material, units: amperes/meter |
| International System of Units (SI) | The International System of Units prescribes the units of measure with symbols and prefixes to form decimal multiples of SI units. The base units are meter (m), kilogram (kg), second(s), ampere (A), kelvin (K), mole (mol), and candela (cd). |
| interpolation | Process of estimating a value which falls between know values, such as the density of a substance at a temperature of 270C when the values at 20o C and 30o C are known |
| Interval velocity | The velocity of waves through a specified thickness interval, typically one formation |
| inverse modeling | A process for determining a model that could produce observed results starting with the observed results |
| inversion | Modeling: the process of determining a possible model from the observed results; Mathematical: finding the quantity whose product with the original quantity will give unity |
| invert | Civil engineering: the lowermost flow line; Mathematical: find the solution to 1 divided by a mathematical object |
| ion | Atom or molecule that has acquired an electric charge by the loss or gain of one or more electrons. |
| IP | Induced polarization |
| iso | Prefix indicating having a constant value, for example, an isobar on a map is a line of constant barometric pressure |
| isobath | A contour of equal depth in a body of water, represented on a bathymetric chart. |
| isostatic anomaly | The Bouguer gravity value that remains after the data have been corrected for the known effects of large scale crust thickness |
| isothermal | Of or indicating equality of temperature. |
| isotherms | Lines connecting points of equal temperature on a weather map. |
| isotopic | Adjective describing atoms of the same atomic mass |
| isotropic | An adjective describing a property that at every point is independent of direction |
| iteration | Repeating a calculation with revised parameter estimates to reduce differences between the calculated result and some reference result |

# J

| JGR | Journal of Geophysical Research, a publication of the American Geophysical Union |
| --- | --- |
| joule (J) | The SI unit of work or energy, defined to be the work done by a force of one newton acting to move an object through a distance of one meter in the direction in which the force is applied. |
| jug | Informal term for a geophone |

# K

| k | Commonly used symbol for wavenumber |
| --- | --- |
| karst | A limestone surface that has been extensively modified by dissolution; characterized by lack of surface drainage, sink holes, and caves |
| kelvin (K) | The SI unit of temperature; a Kelvin temperature is 273.15 higher than the Celsius temperature |
| kick | A sudden change in a variable such as the first break of a seismic trace |
| Klauder wavelet | The basic wavelet produced by the autocorrelation of a Vibroseis sweep |
| Koenigsberger ratio | The ratio of remanent to induced magnetization of a body |

# L

| lag | A time delay between two traces or events |
| --- | --- |
| Lamé's constants | A pair of elastic constants relating stress and strain (λ and μ) |
| Larmour precession frequency | The frequency at which a magnetic dipole precesses about the direction of the applied magnetic field |
| lateral resolution | The minimum horizontal distance between two physical features at which they can be recognized as separate in geophysical data |
| latitude correction | The correction applied to gravity data for the global increase with latitude due to Earth's shape and rotation |
| Lee array | A resistivity array with a fifth electrode in the center, potential differences are measured between the center and the left and right electrodes respectively to determine asymmetry |
| liquid limit | Lowest water content, by weight, of a cohesive soil at which it can behave as a liquid |
| longitudinal conductance | A parameter characterizing the electrical property of a layer which equals the conductivity times the layer thickness |
| longitudinal wave | A wave in which the direction of disturbance is collinear with the propagation direction; Seismic: P wave, a compressional wave |
| looping | A survey method where readings are taken around a loop with periodic return to a previous stations; this is useful for removing temporal variations |
| loss factor | In electromagnetic wave transmission at frequency ω (=2πf) through a material with conductivity σ and dielectric permittivity ε, the loss factor P= σ/ωε , it is used to characterize the rate of wave attenuation |
| Love waves | A surface wave that is a horizontally polarized shear wave |
| low | Electronics: A digital logic state corresponding to a binary "0"; Geophysics: a zone where the parameter measured is distinctly lower than in surrounding zones |
| low-cut filter | A device or operation that removes frequencies below a characteristic frequency from a signal with a controlled transition between passed and cut frequencies; also called a high-pass filter |
| low-pass filter | A device or operation that passes frequencies below a characteristic frequency from a signal with a controlled transition between passed and cut frequencies; also called a high-cut filter |
| low velocity layer | Seismic refraction: a layer that has a lower velocity than the one above so that it does not produce a head wave and is not detected as a first break |
| LVL | Low velocity layer |

# M

| magnetic anomaly | A feature on a magnetic map or profile that is noticeably different from its surroundings |
| --- | --- |
| magnetic declination | Also called magnetic variation, the angle by which magnetic north differs from true (geographic) north |
| magnetic dip | Magnetic inclination |
| magnetic dipole | The simplest unit of a magnetic system consisting of a small current loop, but sometimes visualized as a pair of equal north and south magnetic poles separated by a short distance |
| magnetic dipole moment | The strength of a magnetic dipole; SI unit is amperes/m2; It is the product of the current and the area of the loop and has a vector direction normal to the area of the loop |
| magnetic domain | A region of a grain of magnetic material that has all of the atomic dipole moments aligned |
| magnetic field intensity | H, also called magnetic intensity and magnetizing force; SI unit is ampere-turn/meter a field generated by electrical conduction currents |
| magnetic inclination | The angle the earth's magnetic field vector makes with a horizontal plane, positive is downward. |
| magnetic induction | B, also called magnetic field and magnetic flux density; SI unit is tesla; a field generated by electrical conduction currents and magnetic materials that produces a force on a current or moving electrical charge; related to H by B = μ0(H+M), where μ0 is the magnetic permeability of space and M is the magnetization |
| magnetic moment | Magnetic dipole moment |
| magnetic permeability | The ratio of the magnetic induction to the magnetic field intensity, μ = B/H; in space μ0 = 4π 10-7 weber/ampere-meter |
| magnetic pole | An isolated source or sink of magnetic field which has never been observed in nature, but which is sometimes a convenient conceptualization |
| magnetic storm | A strong disturbance of the geomagnetic field due to high energy particle streams emanating from the sun; they can make magnetic survey data unreliable |
| magnetic susceptibility | The ratio of the magnetization of a body to the magnetic field intensity producing it, k ( or χ) = M/H (dimensionless) |
| magnetization | M, the net magnetic dipole moment per unit volume of a material, units: amperes/meter |
| magnetometer | An instrument for measuring the magnetic field |
| magnetotelluric method | An electromagnetic method for determining the variation of the resistivity of thick layers of the crust by measuring the naturally occurring electric and magnetic field components as a function of frequency |
| master curves | Theoretically calculated families of curves for a range of layered earth models that can be used for interpreting geophysical data, particularly resistivity and electromagnetic |
| MASW | A technique for estimating shear wave velocity as a function of depth by recording surface waves with multiple sensors along a surface line |
| Maxwell's equations | The four basic differential equations that govern electromagnetic fields |
| mega | Prefix indicating 106, one million |
| metal factor | A quantity used in frequency domain induced polarization surveys to represent the apparent resistivity differences at two frequencies to help characterize sulfide ore bodies |
| mho | A unit of electrical conductivity which is equal to 1/ohm; being replaced by the SI unit siemens |
| microgravity | A gravity survey precise to less than 100 microgals |
| migration | A numerical operation applied to seismic (or ground penetrating radar) sections to move the reflections, which may come from dipping reflections or scatterers, into their proper geometrical position on the migrated section |
| milli | Prefix indicating 10-3, one thousandth |
| minimum phase | A seismic wavelet that has the energy concentrated near the beginning resulting in a rapid rise and slower decay, this is approximately the waveform generated by an impulsive source such as an explosive |
| mise-a-la-masse | An electrical survey technique where a conducting body is used as one of the electrodes, the other is placed far away, and the potential is measured at various points on the surface |
| mis-ties | The difference between results obtained at common points by applying the same survey technique along different paths; examples - magnetic values at the point where aeromagnetic survey lines cross or elevation values by leveling around two independent loops |
| mob | Shortened version of "mobilization", the fixed cost associated with preparing for a survey or other equipment use |
| mode conversion | The change in the type of seismic waves that occurs when a wave is obliquely incident on an interface; incident P waves may be partially converted to SV waves or vice versa |
| model | A conceptual representation of a real system that allows theoretical analysis of system responses to various inputs |
| moisture content | For a soil, weight of water divided by weight of solids, reported in percent |
| monopole | An isolated source or sink of magnetic field which has never been observed in nature, but which is sometimes a convenient conceptualization |
| monostatic | Ground penetrating radar: a survey method where a one antenna is used as both the transmitter and receiver |
| multiples | Multiple reflections in seismic reflection or GPR surveys, they are the result of waves that bounce multiple times before being recorded, as opposed to primary reflections which arrive at the detector after a single reflection |
| mute | A numerical operation applied to seismic (or ground penetrating radar) sections to remove a section of the record in which noise overwhelms the signal and thus degrades the final result |

# N

| nanosecond | 10-9 second |
| --- | --- |
| nanotesla | A subunit of magnetic induction B equal to 10-9 tesla, commonly used in magnetic exploration; numerically equal to the older unit gamma |
| natural gamma log | A well log that measures the gamma radiation naturally emitted by the materials penetrated |
| NDT | Non-destructive testing |
| near-field | The zone near a source where longer range approximations may not accurately apply |
| near-surface geophysics | Geophysics applied to the upper zones of the earth, typically the top 100m; sometimes called shallow geophysics |
| neutron log | A well log that uses a neutron source in the logging tool to determine the characteristics of the surrounding material, most commonly hydrogen content, and consequently water content and porosity |
| node | A point of intersection or connection used for measurement or calculation; a point of no motion on a standing wave |
| noise | Any disturbance picked up by a recording system that interferes with observing the desired signal |
| nomograph | A chart for determining the results of a calculation graphically |
| non-polarizable electrode | An electrode used in electrical exploration methods that avoids the electrochemical action occurring when a metal stake is driven placed in the ground; porous pot electrode with a metal contact in a porous container filled with a saturated salt solution of the same metal |
| normal | Geometrical: orthogonal, perpendicular to a surface or line; General: referring to the usual condition |
| normal incidence | A wave or object that strikes an interface perpendicularly |
| normal mode | A vibrational mode for an object or system where all parts vibrate at a common frequency |
| normal move out | The increase in arrival time with increasing distance between the source and the geophone of a seismic wave reflected from a horizontal surface |
| normalize | To adjust data to a norm such as a maximum value of unity or a common mean |
| northing | The distance north of an east-west reference line |
| notch filter | A filter which removes a narrow band of frequencies from a signal, most commonly the electrical power grid frequency of 60 Hz or 50 Hz |
| NSG-SEG | The Near Surface Geophysics Section of the Society of Exploration Geophysicists |
| Nyquist frequency | The maximum frequency that can be contained in a signal if it is going to be accurately sampled for digitizing; Nyquist frequency = 1/(2*sample interval) |

# O

| observer | In geophysics: the person who runs a recording system, such as seismic recording |
| --- | --- |
| octave | A range of a factor of 2 in frequency; up one octave is double the frequency, down one octave is half the frequency |
| oersted | A unit of magnetic field intensity in cgs-emu system |
| off time | For a time-domain electromagnetic survey it is the time after the energizing current is turned off during which secondary fields are measured |
| off-end | A seismic shooting geometry where the source is beyond the end of the geophone spread |
| offset | Seismic: the distance from the source to a particular sensor or sensor group; Electrical: a shift in DC voltage level; Geology; a displacement |
| ohm | The unit of electrical resistance, which equals 1 volt/ampere |
| ohmmeter | A device for measuring electrical resistance |
| ohm-meter | A unit in which the electrical resistivity of a material is measured |
| Ohm's Law | The linear relationship between voltage and current that applies to many common material |
| on time | For a time-domain electromagnetic survey it is the time during which the energizing current flows |
| opacity | The degree to which an object's transparency is diminished; important in 3D visualization |
| open hole | A bore hole or section of a borehole where the walls of the hole are formed by the penetrated rock; not cased |
| optically-pumped magnetometer | A magnetic field measuring instrument that uses the resonant absorption of light by cesium (or rubidium) atom to measure the field |
| optimization | Selecting the system and the controlling parameters to best meet the needs for solving a specified problem |
| optimum window | A seismic reflection technique where the source-to-geophone offset is arranged so that the reflected waves fall in a low noise window between the refraction arrivals and the surface wave arrivals |
| ordnance | Military supplies including ammunition and weapons; of particular environmental concern is ordnance consisting of lost or abandoned bombs, artillery shells and mortar shells |
| orthogonal | Geometrically: perpendicular or normal to a line or surface; Mathematically: functions that are linearly independent |
| outcrop | A section of bedrock that is exposed at the ground surface |
| out-of-phase | A wave that has a phase different from a reference signal; A signal component that is 90 degrees out of phase is sometimes called a quadrature component |
| overburden | The earth material above a level of interest, commonly the unconsolidated layer |
| Overhauser effect |  |
| overvoltage | The decaying voltage that is observed in Induced polarization measurements after the source current is turned off |

# P

| padding | Addition of extra zeros to datasets in order to make larger mathematical transformations (e.g., fast Fourier Transformations). |
| --- | --- |
| paleomagnetism | Study of the change of earth's magnetic field through geologic time. |
| paleoseismology | Spatial, temporal, and magnitude study of prehistoric earthquakes |
| parallax | A change in an object's apparent position resulting from optical refraction; a function of observation angle. |
| paramagnetism | Weak magnetic interactions formed between atoms with incomplete electron shells. |
| Parasnis' method | Method for determining position of thin magnetic sheet. |
| peak acceleration | Ground shaking expressed in terms of percent of gravitational acceleration (i.e., g, where g=9.8 m/s2). |
| peak displacement | Ground shaking expressed in terms of particle displacement (i.e., double integration of the ground acceleration). |
| peak strength | Maximum shear stress achieved from a strained brittle soil. |
| peak velocity | Ground shaking expressed in terms of particle velocity (i.e., integration of the ground acceleration). |
| peg-leg multiple | Multiple reflection having a non-symmetrical travel path. |
| period | Time required for one cycle, oscillation, rotation, etc. |
| permeability | 1. Measure of a fluid's ease of flow through a porous medium, 2. Ratio between the magnetic field and magnetizing force. |
| permittivity | Ratio of the electrical displacement to the electric field strength. |
| Peter's half-slope method | Method for determining depth of magnetic body. |
| phase | (+) Or (-) displacement of a sinusoidal wave from the reference position. |
| phase velocity | Distance traveled per unit time by a point of constant phase. |
| piezometer | Instrument used to measure groundwater head. |
| piping | Groundwater flow and erosion through an engineered structure (e.g. Dam). |
| plastic limit | Moisture content of a cohesive soil that breaks when rolled into a 3mm thread. |
| plasticity index | Moisture range of the plastic state (i.e., the difference of the liquid limit and plastic limit). |
| plume | A distribution that has a shape resembling a feather fanning out from a narrow tip; for example, a contaminant plume is the distribution of contaminated ground water that spreads from a source |
| Poisson's ratio | Ratio of the longitudinal and transverse strains. |
| polarity | 180 degree phase change. |
| polarization | Seismic: direction of particle motion in a shear wave, frequently specified as SV and SH for motions in the horizontal and vertical planes respectively; Electromagnetic waves: the direction of the electric (or magnetic) field vector |
| pole | A single point of special significance, such as the North pole, a mathematical singularity, or one end of a magnet |
| poorly graded | Soil composed of narrow range of particle sizes. |
| poorly sorted | Soil composed of broad range of particle sizes. |
| pore pressure | Pressure of water within a soil's voids. |
| porosity | Ratio of the voids volume to the total volume. |
| potential | Amount of work required to move unit charge, mass, etc. To some position from reference a position. |
| preconsolidation stress | Point marking change in slope on consolidation curve; indicates the value of the largest vertical stress soil has ever experienced. |
| predictive deconvolution | Use of part of a seismic trace to predict and deconvolve another part. |
| predominant period | Period of maximum response from an applied dynamic load. |
| pressuremeter | Field test to measure a soil's in situ stress, compressibility, and strength. |
| pre-stack | Processing operations made prior to trace stacking. |
| primary reflection | Reflection following a direct path to an impedance boundary and back to surface receiver. |
| primary wave (P-wave) | Body wave that elastically compresses and dilates particles in the direction of propagation. |
| principal stresses | Coordinate system containing the maximum and minimum stresses. |
| probabilistic seismic hazard analysis | Seismic hazard calculation that accounts for the uncertainties in earthquake location, earthquake size, and ground motion prediction parameters. |
| propagation | The transmission of wave energy through a medium |
| Proctor compaction test | Method for determining the maximum dry unit weight of a soil. |
| proton-precession magnetometer | Method for measuring earth's total magnetic field; frequency of nuclei precession is proportional to strength of total magnetic field. |
| pseudosection | A graphical representation of a physical property in horizontal distance and depth that resembles a cross section but does not directly represent the properties of the cross section, it typically requires an inversion properties to construct a cross section |
| pseudo-spectral acceleration | Maximum acceleration expressed from a single-degree-of-freedom system; although not true maximum ground motion acceleration, close approximation. |
| pseudo-spectral velocity | Maximum velocity expressed from a single-degree-of-freedom system; although not true maximum ground motion velocity, close approximation. |
| psuedostatic method | Slope stability evaluation that idealizes dynamic seismic load as horizontal static force. |
| pull-up | Localized, shallow high-velocity zone "pulling up" reflection and creating structural artifact. |
| pulse | Single, short duration wavelet. |
| pulse transient | IP measurement that evaluates overvoltage as function of time. |

# Q

| Q | Quality factor |
| --- | --- |
| Quad/Quad | A two-boat system with an array of 4 streamers and 4 sources to efficiently acquire 3D marine seismic data. |
| quadrature | Signal component out of phase. |
| quality factor | Q, Coefficient used in characterizing the intrinsic attenuation of seismic waves. |
| quick clays | Highly sensitive clays (i.e., undergo strength reduction with disturbance). |
| quicksand | Upward groundwater seepage that creates an effective stress, thus strength, close to zero. |
| Q-wave | Seismic surface wave most often referred to as the Love wave. |

# R

| radar | Exploration system that transmits and receives electromagnetic waves. |
| --- | --- |
| radiation damping | Stress wave amplitude loss due to the spreading of energy over larger volumes; sometimes called geometrical spreading. |
| random noise | Incoherent background energy. |
| Rankine's theory | Approach to the lateral earth pressure problem. |
| ray path | Line drawn outward from seismic source that intersects wavefront at a right angle in an isotropic medium. |
| Rayleigh wave | Surface wave with an elliptical orbit oriented in a vertical plane; motion diminishes with depth. |
| reciprocity | Equivalent travel time for symmetrical forward and reversed refraction profile. |
| recurrence rate | Empirical relationship describing the average rate at an earthquake of some size will be exceeded; characterized for each seismic source zone. |
| reflection | Return of a wavefront after encountering an impedance boundary; the properties of the boundary will determine reflected wave characteristics. |
| reflection coefficient | Ratio of the amplitude of the incident and reflected wave; values of amplitudes are a function of angle of incidence, density and seismic velocity of horizons. |
| refraction | Bending of wavefront (or ray path) as it propagates across impedance boundary. |
| regional trend | Bouguer anomalies, produced from large features, which remain smooth over considerable distances. |
| relative compaction | Comparison of soil's actual dry density to the maximum dry density determined by a Proctor test. |
| relative density | Ratio of the difference between the maximum and actual void ratios, and the difference between the soil's maximum and minimum void ratios. |
| remnant magnetization | Existing magnetism in a rock independent of the existing polarizing field. |
| residual anomalies | Remaining gravity anomaly after regional anomalies are removed; a function of local disturbances. |
| residual soils | Soil formed from parent rock on site. |
| residual strength | Sustainable shear stress in a material after substantial strain. |
| resistivity | Physical constant, along with length and cross-sectional area, of the substance used to construct resistor. |
| resolution | The ability to spatially and temporally detect/image/separate individual features. |
| resonance | Harmonic loading applied at a medium's natural period. |
| resonant column test | Common laboratory test used to determine low-strain dynamic properties of a soil. |
| response spectrum | Describes maximum response of a single-degree-of-freedom system to a particular input motion as function of natural period and damping. |
| Richter magnitude | Earthquake magnitude scale used in southern California to describe shallow earthquakes with epicentral distances less than 600 km. |
| Ricker wavelet | An idealized zero-phase seismic wavelet that is the second time derivative of the normal error function |
| rigidity modulus | See shear modulus |
| rippability | Capacity of earth to be excavated without drilling or explosives; often described in terms of compression wave velocity |
| rise time | The end time of a fault rupture. |
| RMS velocity | Root mean square velocity - an average velocity determined by taking the mean of the square of the velocity for each time interval, then taking the square root of the result; the stacking velocity derived from normal moveout analysis is sometimes used as an approximation for the RMS velocity. |
| rock quality designation (RQD) | Percentage of rock core pieces longer than 100 mm; useful measure of fracturing, thus stability. |
| rock susceptibility | The degree to which a rock can be magnetized. |
| roll along | A procedure for utilizing a number of detectors and/or energy sources to collect data along a long line; equipment from one end of the line is moved to the far end to advance along the line; common in seismic reflection |
| Rossi-Forel intensity | An earthquake intensity scale. |
| rupture | Fracture |
| R-wave | An abbreviation for Rayleigh wave |

# S

| SAGEEP | Symposium on the Application of Geophysics to Engineering and Environmental Problems is the primary annual meeting of EEGS |
| --- | --- |
| salinity | A measure of the dissolved solids in water, commonly in grams per kilogram |
| sample interval | The time interval at which a signal is sampled for digitizing an analog signal; e.g., a 1 millisecond sampling interval would result from uniformly sampling a signal at a 1 kilohertz rate. |
| sample log | A graphic representation of the lithologies penetrated by a well as observed from cutting samples. |
| sample rate | The number of times per second that the value of an analog signal is measured in the digitization process; the sample rate is the reciprocal of the sample interval. |
| sampling function | A mathematical function of infinite length with values of 1 at equally spaced increments and values of 0 everywhere else; also called a comb function. |
| SASW | Spectral Analysis of Surface Waves, a technique for measuring surface wave properties at a set of increasing spacings on the surface of the Earth which are inverted to determine shear velocity as a function of depth |
| saturation | The extent to which a parameter attains its maximum allowable value; for example, when water fills 70% of the pore space in a rock the water saturation is 0.7; for electrical signals it is the condition where the signal amplitude reaches the maximum level a circuit will handle. |
| scaling | Adjusting the size by some proportionality factor |
| scattering | The irregular redirection of wave energy (electromagnetic or seismic) due to non-uniformities in the propagating medium. |
| Schlumberger array | An electrode configuration used in a 4-point electrical resistivity measurements where the inner electrodes which measure the electrical potential difference are relatively closely space compared to the distance to the outer current electrodes. |
| Schmidt net | An equal-area plot for geological data using the Lambert equal-area projection. |
| scintillometer | A device for measuring radioactive emissions which employs a material that emits flashes of light when struck by a gamma ray, x ray, or high energy particle. |
| secondary arrival | A wave, usually referring to seismic, from a specific boundary that arrives at a detector after the first arrival signal. |
| secondary compression index |  |
| secondary voltage | A voltage measured in a detector: a) [for induced polarization surveys] after the primary current is turned off, or b) [in electromagnetics surveys] due to secondary magnetic field produced by current flow in the subsurface. |
| secondary wave (S-wave) | Shear wave (seismic) |
| second-derivative map | A map of a field property over an area that is constructed by taking the second derivatives of the field map; this may represent horizontal or vertical derivatives. |
| SEG format | A series of data formats recognized by the Society of Exploration Geophysicists for the purpose of standardizing the format of geophysical data for recording and transfer. |
| seiche | The large-scale oscillatory motion of a body of water in a basin. The period is determined by the dimensions and shape of the basin. |
| seismic anisotropy | Differences in the velocity of seismic waves in a rock unit depending on the direction of the wave propagation and the type and polarization of the waves. |
| seismic discontinuity | A sharp (relative to the relevant seismic wavelengths) discontinuity in the velocity of propagation of seismic waves at some surface in the material. |
| seismic facies | A zone observed within a set of seismic sections that have a distinctive pattern of reflections that can be correlated across some area and frequently relate to a consistent depositional pattern. |
| seismic gap | A portion of an active fault line which has not had an earthquake as recently as the surrounding sections of the fault. This may be an indication of a zone with built up stress that is likely to produce an earthquake relatively soon. |
| seismic hazard | A fault that has the potential to move and produce a significant earthquake. |
| seismic hazard assessment | The study of potential earthquake zones to evaluate which have the greatest risk of producing damaging earthquakes. |
| seismic impedance | Acoustic impedance |
| seismic risk | The risk of an earthquake producing various levels of damage in an area. |
| seismic wave | A mechanical wave that propagates through the earth or along its surface. It has many forms including p-wave, s-wave, and surface waves. They may be produced by earthquakes, volcanic activity, or mechanical devices. |
| seismicity | The earthquake activity of a region. |
| seismites | A geological deposit produced by earthquake activity. |
| seismology | The study of seismic wave phenomena; natural and man-made. |
| self potential (spontaneous potential) | An electrical potential (voltage) difference that occurs in the earth due to naturally occurring processes, most commonly electrochemical action or groundwater flow. It is also the name given to a geophysical technique that utilizes these potential differences to study the subsurface. |
| semblance | A statistical measure of the similarity of seismic traces within a set. |
| sensitivity | A measure of the smallest signal that can be clearly measured by a sensor. |
| settlement |  |
| shadow zone | A zone on the surface of the earth where a particular signal is not observed due to the propagation characteristics of the materials directing the energy away from the shadow zone; most commonly refers to the zone where P waves from an earthquake are not observed due to refraction at the core-mantle boundary. |
| shaping deconvolution | A mathematical technique to modify the shape of a seismic wavelet within a seismic trace; a common use is to make the wavelet zero phase. |
| shear | A deformation of a material where the displacement of the material is perpendicular to a set of planes with equal deformation; slipping a deck of cards is a simple model. |
| shear modulus | The proportionality constant between shear stress and shear strain that characterizes a material within the elastic range. |
| shear strain | A measure of the relative perpendicular displacement of points in a material relative to the distance through the material; it is the ratio of perpendicular displacement per unit of distance through the material, it is inherently dimensionless being a ratio of length to length. |
| shear strength | The maximum shear stress to which a material can be subjected before it fractures. |
| shear stress | The difference in perpendicular shearing forces on a body per unit of distance through the material. |
| shear wave | A seismic wave that is propagated due to shear stresses in the material; the displacement of the particles is perpendicular to the direction of propagation. They can have two polarization directions perpendicular to the propagation direction. Also called S waves because they are the second type of wave to arrive from an earthquake. |
| shear-wave splitting (birefringence) | The effect of the splitting of a shear wave into two parts with different polarizations due to velocity differences that are polarization dependent. |
| sheet pile | A corrugated steel sheet that is driven into the ground to form a vertical barrier. |
| Shelby tube sampler | A steel tube that is driven into the ground to penetrate the soil and then pulled out with a column of the soil trapped in its interior. The column of soil is examined for soil properties. |
| shielding | In electrical instrumentation - a metallic layer surrounding an electrical system to prevent interference by external electric and or magnetic fields. |
| shingling | A pattern, such as in a sedimentary deposit with a sequence of overlapping layers like roof shingles; also imbricate or echelon pattern. |
| shock wave | A wave of high amplitude and short duration |
| shootback method | An electromagnetic survey technique designed for hilly terrain in which a transmitter and receiver are both used at each station location. |
| shot | An input of seismic energy, originally with explosives, but now commonly used for any type of user initiated input, particularly impulsive energy sources. |
| shotpoint | The location of a seismic energy input. Also called source point, particularly for non-explosive sources. |
| shrinkage | A reduction in size |
| SH-wave | A shear wave which is polarized with its plane of particle motion perpendicular to the vertical plane. |
| side-swipe | Seismic (or other types) reflection energy that is detected from reflections that occur out of the vertical plane that contains the source and detector. |
| signal | The desired field (seismic, electrical, radar, etc.) That is detected by a recording system. |
| signal-to-noise ratio | The ratio of the signal strength to the strength of other fields that interfere with observing the desired signal; it may be expressed as an amplitude ratio or as a power ratio. |
| sink | In electrical surveys - an electrode at which current is withdrawn from the earth, as opposed to the source at which current is injected. In geology a place in karst topography where water drains from surface into underground conduits, more commonly sinkhole. |
| site response | Features of a particular study site that characterize the background response to the instruments used in a geophysical survey. |
| skin depth | In electromagnetic techniques - the depth to which the EM fields penetrate as measured by the depth where the fields drop to 1/e (about 37%) of the value at the surface. |
| skin effect | The effect that very high frequency electromagnetic field propagate along the surface of a conductor. |
| SLAR | Side-looking Airborne Radar (a trade name). |
| smoothing | A mathematical technique for reducing noise and local variability in data by taking a running weighted average of a data point and its neighbors. |
| Snell's law | The law of refraction. At an interface the direction of propagation of a wave changes according to the equation; sine i /sine r = Vi/Vr or = nr/ni, where i and r are the angles the incident and refracted rays respectively make with the normal to the interface, Vi and Vr are the wave propagation speeds in the 2 media and ni and nr are the indices of refraction in the 2 media. |
| soil | The unconsolidated material on the earth's surface that is a mix of mineral and organic matter. Regolith. |
| soil improvement | Changing the characteristics of a soil for some desired purpose such as agriculture or construction. |
| soil nailing |  |
| sonar | Sound Navigation and Ranging (acronym); a technique for using reflected sound waves to image objects and irregularities in the transmitting material. |
| sonde | A devise containing sensors and/or energy sources that is placed in or drawn through a region to determine its properties; commonly used in boreholes, oceanography and atmospheric studies. |
| sonic | An adjective referring to things that depend on sound or acoustic phenomena. |
| sonic log | A well logging device that measures the travel time of high-frequency acoustic waves between sources and receivers on a sonde for the purpose of a detailed determination of seismic velocities as a function of depth. |
| sorption | The removal of specific types of atoms or molecules from a fluid by attachment to surfaces on grains of a porous material. |
| Sosie | A seismic technique that utilizes a sequence of pseudo-random energy inputs to observe reflections by cross-correlation of recorded data with a record of the input pulses. ( a trademark) |
| source | A device that puts some form of energy in the ground or naturally produces some type of measurable field; examples: seismic - explosive, hammer, vibrator; electrical - an electrode connected to the positive side of a power supply; gravity - an intrusive body that produces a measurable, anomalous gravity field. |
| S-P interval (time) | The time interval between the arrival of the P and S waves from a common source, usually from earthquakes. |
| sparker | A seismic energy source that generates seismic waves by a high-voltage spark in water. |
| spatial aliasing | The effect of incorrectly representing data when the sampling interval is too long for the variability of the measured property. The effect makes high wavenumber components misrepresented as lower wave numbers. |
| spatial frequency | The periodic frequency of a variable in space, such as might be expressed in cycles per kilometer. Example: the number of ocean waves in a one-kilometer profile of a section of open ocean. |
| specific gravity | The density of a substance relative to the density of water; effectively equivalent to density in grams per cubic centimeter. |
| spectrum | The distribution of energy as a function of frequency of wavelength for a signal. Example: the intensity of a rainbow for the various colors. |
| spherical divergence | The decrease in the amplitude (or power) of a wave as it expands in 3 dimensions, which would be energy spread over a sphere in a homogeneous medium. |
| spheroid | A geometrical shape which is like a flattened (or elongated) sphere, the cross sections taken perpendicular to the rotational symmetry axis are circular, cross sections perpendicular to this are elliptical. |
| spiking deconvolution | A mathematical procedure for modifying a seismic data trace by altering the original wavelet into a shorter, highly localized wavelet (spike). |
| split spoon sampler |  |
| split spread | A spread of seismic detectors for which some portion are on opposite sides of the energy source. |
| stack | Verb: to add together time sequence data records that are considered to have the same signal content but different noise content to improve the signal-to-noise ratio. Noun; the data sequence resulting from the stacking process. |
| stacking chart | A chart used in seismic reflection surveys to decide which traces will be stacked for each bin, i.e., to make a composite trace. |
| stacking velocity | Seismic or radar: The velocity used in the normal moveout correction which best removes the normal moveout effect. |
| stake resistance | The electrical resistance between a stake electrode and the ground. |
| standard penetration test |  |
| standing wave | A wave that is formed by the interference of oppositely traveling waves and results in a wave pattern that is fixed along the line of propagation |
| static correction | A correction applied to seismic data to remove the effect of travel time differences through near surface materials that would otherwise distort the image of subsurface structures. |
| static shift | A time shift of a seismic trace which applies equally to each data point in the trace. A static correction is normally applied to remove the effect. |
| step function | A mathematical function where the values are constant at some value, typically 0, before a selected time T and are constant at a new value, typically 1, at T and later in time. |
| step out | The difference in arrival time from trace to trace of an identifiable event on a seismic section. |
| stereographic projection | A projection used to represent polar directions by the projection from a southern hemispherical surface onto an equatorial plane. |
| stickogram |  |
| stochastic | Adjective: involving a degree of randomness. |
| Stoneley wave | A seismic surface wave that propagates along the boundary between two media, similar to a Rayleigh wave on a free surface. |
| strain | The fractional deformation of a material due to applied stress. |
| streamer | A seismic device for towing a long string of sensors in the water behind a ship or less commonly on land behind other vehicle. |
| strength | The maximum stress before a material fails. |
| strike | The compass direction of the line of projection on a horizontal surface of a geological or manmade feature such as a dipping rock layer or a pipeline. |
| strong-motion seismology | A seismograph for recording waves from nearby earthquakes |
| superposition | The principle where the net wave or field involving several components is the simple algebraic sum of the amplitudes of the components at each time and place. |
| suppressed layer | In resistivity surveys: a thin layer which does not have sufficient resistivity contrast with the layers above and below may not be identifiable on a resistivity sounding curve. |
| surcharge |  |
| surface conductivity | Electrical conduction along the surface of mineral grains. |
| surface wave | A wave that travels along the surface of the earth; examples: Rayleigh waves and Love waves. |
| SV-wave | A seismic shear wave which is polarized in the plane formed by the local vertical and the wave propagation direction. |
| sweep | The range of frequencies of a vibratory energy source that gradationally changes frequencies with time. |
| synthetic seismogram | A calculated seismic trace that is generated by convolving an acoustic impedance log determined from a well or model with a representation of the seismic wavelet. This is commonly used to relate seismic traces to borehole litho logic information. |

# T

| takeout | An electrical connector on a multichannel signal cable where a sensor, such as a geophone, can be connected to a pair of wires in the cable. |
| --- | --- |
| telluric currents | A naturally-occurring low-frequency electrical current that flows over extended regions of the earth. |
| telluric method | An electrical exploration method used to map lateral resistivity anomalies in the earth on the basis of disturbance of the telluric current patterns. |
| terrain correction | A correction applied to gravity data to remove the effects of topographic features in the vicinity of the gravity station. |
| thermoremanent magnetization | The remanent magnetization that occurs in a rock when it cools to below the Curie point in an ambient magnetic field. |
| thin layer | A layer of the earth that is near or below the vertical resolution of the method being used. |
| tidal effects | The time-varying effects on gravity measurements of the attractions of Sun and Moon and the yielding of the solid earth and its oceans under these attractions. |
| time section | A seismic cross section where the vertical axis is presented in seismic travel time. |
| tomography | A mathematical technique for inferring the distribution of the properties in a 2- or 3-D region by making a dense series of measurements of waves or fields passing through the region along multiple intersecting paths. For example: determining the seismic velocity distribution from travel times between a series of sources and detectors in parallel bore holes. |
| topographic effects | Effects caused by differences in elevation, these occur in varying degrees for most geophysical methods |
| torsion balance | A device that uses a pair of masses on a beam suspended from a torsion fiber to measure the gradient of the gravitational field. |
| total-field anomalies | Anomalies of the magnetic field determine from data acquired using a total field magnetometer. |
| totally reflected | A wave, seismic or electromagnetic, is totally reflected from an interface when it is incident at an angle exceeding the critical angle; the wave velocity must increases across the interface for this to occur. |
| transient | A disturbance which has a limited extent in time after a change occurs to the system. |
| transillumination | Lit from behind by transmission |
| transmission coefficient | The ratio of the amplitude of a transmitted wave across an interface to the amplitude of the incident wave. Sometimes it may be expressed as an intensity ratio. |
| transmitter | A devise that emits an electrical or electromagnetic signal for geophysical exploration purposes |
| transmissivity | Hydrology: the product of an aquifer layer thickness times the hydraulic conductivity; it is a measure of how well an aquifer transports water. |
| transverse wave | A wave where the direction of the disturbance is perpendicular to the direction of wave propagation; examples: electromagnetic waves in a nonconducting medium or seismic shear waves. |
| travel-time curve | A graph of the arrival time as a function of distance from the source. |
| trend surfaces | A mathematical surface that represents a smoothed version of a data map for the purpose of separating large-scale effects from localized anomalies. |
| triaxial test |  |
| tsunami | A long wavelength ocean wave generated by large-scale ground motions such as due to an earthquake; sometimes improperly called a tidal wave. |
| tube wave | A mechanical wave that propagates along a borehole. |
| Turam profile | An electromagnetic survey method that uses two search coils a fixed distance apart to measure the variations in the field produced by a large current loop on the surface of the ground (or a long insulated wire with grounded ends). |
| two-way travel time | The time for a wave to travel from a surface source to a reflecting interface and back to a detector on the surface. |

# U

| uncertainty | A measure of the precision of a measurement based on statistical measures |
| --- | --- |
| unconformity | A surface of erosion or non-deposition between adjacent rock layers |
| unijunction transistor | Specialized transistor often used in oscillator circuits |
| unit circle | Circle of unit radius used for graphical interpretation of trigonometric functions and frequency response |
| Universal Transverse Mercator | Geographical (map) coordinate system using metric measure (meters) |
| update | A correction of a measurement or system to a newer standard |
| uphole geophone | Geophone at top of borehole |
| uphole seismic | Seismic observations with receiver in borehole and source t at surface or vice versa |
| uphole shooting | Uphole seismic with source in the borehole. |
| uphole survey | A series of seismic velocity measurements in a borehole using a downhole source and surface geophones, usually to determine near-surface velocities |
| uphole time | Seismic travel time for uphole shooting |
| upward continuation | Computation of potential field for a higher altitude based on survey data from a lower altitude |
| UTM | Abbreviation for Universal Transverse Mercator |
| UXO | Acronym for UneXploded Ordnance |

# V

| Va | Variable area trace display |
| --- | --- |
| V2 | Wave velocity in the second layer |
| vacuum-tube voltmeter | Voltmeter using vacuum tubes to achieve a high input impedance |
| vadose zone | The unsaturated soil zone above the water table |
| variable-area | Time trace display with wiggly lines with one polarity blackened in |
| variable density | Time trace display with trace value indicated by grey scale. |
| variance | Standard deviation squared |
| variometer | Magnetometer which detects change of direction of magnetic field |
| V-band | Electromagnetic wave frequencies in the range 46-56 GHz |
| V-bar | Average velocity ( the sum of the distances/the sum of the times) |
| vectogram method |  |
| vector | Physical quantity having magnitude and direction |
| velocity | Distance traveled per unit time |
| velocity analysis | Analysis of data in terms of wave velocity |
| velocity filter | Two dimensional filtering of seismic data according to wave velocity |
| velocity focusing | Variations in velocity which cause seismic rays to converge |
| velocity function | Wave velocity as a function of position (usually depth) |
| velocity inversion | Computation of wave velocity from data |
| velocity log | Sonic velocity compiled as a function of depth, also called sonic log |
| velocity profile | A cross section indicating wave velocity in distance and depth |
| velocity spectrum | Velocity of reflected waves as a function of two way travel time |
| velocity survey | A survey carried out to determine wave velocity |
| Venn diagram | A two dimensional logic diagram indicating combination of sets of information |
| vernal equinox | The time in the spring when the daytime and nighttime are of equal length |
| vernier | Scale for reading distances or angles that allows direct reading of subdivisions |
| vertical electric sounding | Resistivity measurement designed to determine variation of resistivity with depth, commonly abbreviated as VES |
| vertical exaggeration | The vertical scale on a cross section is some multiple of the horizontal scale. |
| vertical intensity | The vertical component of the Earth's magnetic field |
| vertical loop dip angle method | Electromagnetic measurement which determines the dip of the ac magnetic field |
| vertical profile | A record of changes in an Earth property with depth |
| vertical stack | Adding together several traces with approximately the same source and detector locations to enhance signal relative to random noise |
| vertical time | Time for a wave to travel a vertical distance (and back. Optional) |
| VES Vibroseis | Vertical Electric Sounding As seismic energy source using a hydraulically driven vibrator (TM Conoco) |
| viscoelastic | Property of a material which is both viscous and elastic |
| viscometer | A device for measuring viscosity |
| viscosity | The mechanical resistance to a shearing velocity |
| viscous magnetism | Magnetization acquired over a long period of time in the presence of the Earth's field |
| VLF | Very Low Frequency (electromagnetic signals in the 10 KHz to 30 KHz range) |
| voice grade | A communication channel or circuit capable of passing intelligible voice signals (~500 to 5000 Hz). |
| void ratio | Ratio of volume of voids in a soil to volume of solids |
| voltmeter | A device for measuring voltage (or potential difference). |
| volume magnetization | Magnetic dipole moment per unit volume |
| volume reverberation | Reverberation within a volume, typically a volume of water (not reverberation from the surface and bottom) |
| vortex shedding | Eddies created downstream when a fluid (air or water) passes a long narrow object. |

# W

| walkaway | Test of signals created from a stationary source and receivers placed as successively greater distances |
| --- | --- |
| Warburg impedance |  |
| Warburg region |  |
| water cut | Fraction or percent of water produced when pumping petroleum from a given formation |
| water injection | A method of driving petroleum to another well by injecting water into the first well |
| water saturation | Percent or fraction of water compared to the maximum amount that can be contained in a formation |
| water velocity | Signal (wave) velocity in water |
| waterbreak | First arrival of a signal through the water layer |
| wave | A roughly sinusoidal signal in a medium such as air, water or rock. |
| wave equation | An equation expressed in partial derivatives. One of its solutions is a wave. |
| wave impedance | Ratio of cause to effect (strain to stress for seismic, electric to magnetic field for electromagnetic waves) |
| wave spreading | Reduction in intensity of a wave due to spreading or divergence |
| wave tilt | Tilt of an otherwise plane electromagnetic wave, usually due to due to subsurface conductors |
| wave train | A wavelet with a tail (coda) due, for example, to reverberation within thin layers. |
| waveform | The time or distance trace of a wave. |
| wavefront | A surface which connects the parts of a wave that have the same phase |
| wavefront chart | A graphical representation of spreading wavefronts, showing their shape as they pass through a horizontally layered earth |
| waveguide | Geological or manmade structures which keep waves contained and guided |
| wavelength | Distance between consecutive crests for a sinusoidal wave |
| wavelet | The fundamental waveform contained in a waveform representing reflections or refraction from multiple layers |
| wavenumber | 2*pi/wavelength |
| wavenumber filtering | Filtering wave data according to the wavenumber (requires multiple traces) |
| weathering | Near-surface rock becomes fragmented due to interaction with surface water, freezing conditions, wind, etc. |
| weathering correction | Time compensation to account for slower seismic velocity in the weathered layer |
| weber | Measure of magnetic flux |
| weight-drop | A seismic energy source that produces waves by the impact of a falling object with the ground, where the rate of fall is enhanced by a mechanical or pneumatic device |
| weighted array | An array of detectors in which the contribution of the individual detectors is adjusted to produce a desired effect (such as beam steering) |
| weighted average | The average signal produced by summing the contribution of the individual signals which are adjusted to produce an improvement in the total signal |
| well-graded | Adjective describing unconsolidated earth material that has a wide rang of grain sizes so that finer particles fill spaces between coarser particles |
| well log | Measurements or observations made in a borehole and recorded as a function of depth |
| well shooting | Seismic observations with receiver in borehole and source t at surface or vice versa |
| well-sorted deposit | A sedimentary deposit with a small range of particle sizes |
| well tie | Using information from wells to interpret or verify surface geophysical measurements |
| well velocity survey | Interpretation or summary made from wave velocities measured wells |
| Wenner array | An array for electrical resistivity in which four electrodes are placed in a line with equal distances between the electrodes. |
| westing | In land surveying, the distance west from the survey reference point |
| white noise | Noise in which the signal strength at all frequencies is equal (refer to concept of white light) |
| whiten | Filtering signals so that the signal strength at all frequencies is equal |
| wide angle reflection | The separation between source and receiver for WAR is greater than the depth to the reflecting surface. |
| Wiener filter | A Wiener filter converts a given signal into a desired signal |
| wiggle trace | A time-trace of a signal that is plotted as instantaneous value vs. Time. A conventional plot. (c.f. Variable density and variable area plots) |
| wind noise | Acoustic noise due to wind (interferes with seismic signals) |
| wind scale | A numerical scale expressing wind intensity |
| window | A region of times or distances. A larger data set is windowed to produce a subset of the data. |
| wireline log | A well log acquired by deploying instruments in a borehole. The instruments are suspended by a wire. |
| witness marker | A permanently located marker. |
| word | In computer usage, digital information comprised of a small number of bits. Usually eight bits. |
| Worden | A gravity meter made by the Worden company |
| wow | Distortion in recording caused by a slow variation in recording speed. A signal with a constant pitch sounds like the word "wow" on playback. |
| WWV | US Radio station broadcasting a time signal (Carrier is at 5 and 10 MHz and other frequencies.) |
| Wx | Abbreviation for weather |
| Wyllie relationship | A simple weighted average used to predict the physical properties of mixtures |
| Wyrobek method | A seismic refraction interpretation method for unreversed profiles that uses delay times |

# X

| X | Horizontal distance |
| --- | --- |
| X2-T2 analysis | A method for finding wave velocity for reflected signals using the squares of the distances and travel times |

# Y

| yardstick | A ruler three feet long. A standard reference. |
| --- | --- |
| yaw | Rotation around a vertical axis. Refers to an airplane or ship departing from a forward course. |
| Young's Modulus | the proportionality constant between the one-dimensional stress and strain of an elastic body |
| yo-yo | Repetitive up and down motion (named for the motion of a toy) |

# Z

| Z/A | Atomic number divided by atomic mass |
| --- | --- |
| Zener diode | A semiconductor diode which, when reverse biased, maintains a constant voltage. Used to regulate voltage in a power supply circuit. |
| zenith | Directly overhead |
| zero frequency | Direct current (strictly). Signal of extremely low frequency (~ < 1 Hz). |
| zero frequency seismology | Study of earth displacements which occur at "zero frequency", such as tilt. |
| zero-lag correlation | The value of a cross or auto correlation function at zero lag |
| zero phase | A signal (e.g., a wavelet) for which all frequency components have zero phase. (The signal must be symmetric about time zero.) |
| zeta potential | An electrical potential produced by ion adsorption at the boundary of a liquid and a solid |
| Zoeppritz equations | Equations expressing seismic signal reflection and refraction amplitudes as a relation of angle of incidence and elastic constants. |
| Zone chart | Chart for correction gravity observations for topography near the measurement station. |
| z-plane | The plane for plotting complex numbers |
| z-transform | A time varying signal is expressed as coefficients of a polynomial using the variable z. A compact method of writing the frequency response of a time varying signal. |